| TITLE: SHIP/SHORE SAFETY CHECK LIST | FORM NO: | ## TZ 069 |
| --- | --- | --- |
| REVISION: 2 | DATE:05/10/2024 |  |

| Ship’s Name: | SOFI I |
| --- | --- |
| Date and time: | 07/28/2024 |
| Port and berth: | NEGIN ISLAND / JETTY #21 |
| Terminal: | SPM CO. |
| Cargo to be transferred: | HYDROCARBON |

Checks pre-arrival

| Part 1A. Tanker: checks pre-arrival |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 1 | Pre-arrival information is exchanged (6.5, 21.2) |  |  |
| 2 | International shore fire connection is available (5.5, 19.4.3.1) |  |  |
| 3 | Transfer hoses are of suitable construction (18.2) |  |  |
| 4 | Terminal information booklet reviewed (15.2.2) |  |  |
| 5 | Pre-berthing information is exchanged (21.3, 22.3) |  |  |
| 6 | Pressure/vacuum valves and/or high velocity vents are operational (11.1.8) |  |  |
| 7 | Fixed and portable oxygen analyzers are operational (2.4) |  |  |

| Part 1B. Tanker: checks pre-arrival if using an inert gas system |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 8 | Inert gas system pressure and oxygen recorders are operational (11.1.5.2, 11.1.11) |  |  |
| 9 | Inert gas system and associated equipment are operational (11.1.5.2, 11.1.11) |  |  |
| 10 | Cargo tank atmospheres’ oxygen content is less than 8% (11.1.3) |  |  |
| 11 | Cargo tank atmospheres are at positive pressure (11.1.3) |  |  |

| Part 2. Terminal: checks pre-arrival |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 12 | Pre-arrival information is exchanged (6.5, 21.2) |  |  |
| 13 | International shore fire connection is available (5.5, 19.4.3.1, 19.4.3.5) |  |  |
| 14 | Transfer equipment is of suitable construction (18.1, 18.2) |  |  |
| 15 | Terminal information booklet transmitted to tanker (15.2.2) | N/A |  |
| 16 | Pre-berthing information is exchanged (21.3, 22.3) |  |  |

# Checks after mooring

| Part 3. Tanker: checks after mooring |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 17 | Fendering is effective (22.4.1) |  |  |
| 18 | Mooring arrangement is effective (22.2, 22.4.3) |  |  |
| 19 | Access to and from the tanker is safe (16.4) |  |  |
| 20 | Scuppers and save-alls are plugged (23.7.4, 23.7.5) |  |  |
| 21 | Cargo system sea connections and overboard discharges are secured (23.7.3) |  |  |
| 22 | Very high frequency and ultra-high frequency transceivers are set to low power mode (4.11.6, 4.13.2.2) |  |  |
| 23 | External openings in superstructures are controlled (23.1) |  |  |
| 24 | Pumproom ventilation is effective (10.12.2) |  |  |
| 25 | Medium frequency/high frequency radio antennae are isolated (4.11.4, 4.13.2.1) |  |  |
| 26 | Accommodation spaces are at positive pressure (23.2) |  |  |
| 27 | Fire control plans are readily available (9.11.2.5) |  |  |
| Add.1 | Personnel in charge of cargo operations are aware of actions in the case of failure of the inert gas plant. |  |  |
| Add.2 | Emergency towing-off pennants are correctly rigged and positioned |  | Terminal requirement for use of towing-off pennants to be adhered |
| Add.3 | Window/split type air conditioning units are disconnected |  |  |

| Part 4. Terminal: checks after mooring |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 28 | Fendering is effective (22.4.1) |  |  |
| 29 | Tanker is moored according to the terminal mooring plan (22.2, 22.4.3) |  |  |
| 30 | Access to and from the terminal is safe (16.4) |  |  |
| 31 | Spill containment and sumps are secure (18.4.2, 18.4.3, 23.7.4, 23.7.5) |  |  |

Checks pre-transfer

| Part 5A. Tanker and terminal: pre-transfer conference |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Check | Tanker status | Terminal status | Remarks |
| 32 | Tanker is ready to move at agreed notice period (9.11, 21.7.1.1, 22.5.4) | A |  |  |
| 33 | Effective tanker and terminal communications are established (21.1.1, 21.1.2) |  |  |  |
| 34 | Transfer equipment is in safe condition (isolated, drained and de-pressurized) (18.4.1) |  |  |  |
| 35 | Operation supervision and watchkeeping is adequate (7.9, 23.11) |  |  |  |
| 36 | There are sufficient personnel to deal with an emergency (9.11.2.2, 23.11) |  |  |  |
| 37 | Smoking restrictions and designated smoking areas are established (4.10, 23.10) |  |  |  |
| 38 | Naked light restrictions are established (4.10.1) |  |  |  |
| 39 | Control of electrical and electronic devices is agreed (4.11, 4.12) | A |  |  |
| 40 | Means of emergency escape from both tanker and terminal are established (20.5) |  |  |  |
| 41 | Firefighting equipment is ready for use (5, 19.4, 23.8) |  |  |  |
| 42 | Oil spill clean-up material is available (20.4) |  |  |  |
| 43 | Manifolds are properly connected (23.6.1) |  |  |  |
| 44 | Sampling and gauging protocols are agreed (23.5.3.2, 23.7.7.5) | A |  |  |
| 45 | Procedures for cargo, bunkers and ballast handling operations are agreed (21.4, 21.5, 21.6) | A |  | - bunkers operation not allowed - during operation vessel should have upright |
| 46 | Cargo transfer management controls are agreed (12.1) | A |  |  |
| 47 | Cargo tank cleaning requirements, including crude oil washing, are agreed (12.3, 12.5, 21.4.1) | A | N/A | See also parts 7B/7C as applicable |

| Part 5A. Tanker and terminal: pre-transfer conference (cont.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Check | Tanker status | Terminal status | Remarks |
| 48 | Cargo tank gas freeing arrangements agreed (12.4) | A | N/A | See also part 7C |
| 49 | Cargo and bunker slop handling requirements agreed (12.1, 21.2, 21.4) | A | N/A | See also part 7C |
| 50 | Routine for regular checks on cargo transferred are agreed (23.7.2) | A |  |  |
| 51 | Emergency signals and shutdown procedures are agreed (12.1.6.3, 18.5, 21.1.2) | A |  |  |
| 52 | Safety data sheets are available (1.4.4, 20.1, 21.4) |  |  | As per shipper msds |
| 53 | Hazardous properties of the products to be transferred are discussed (1.2, 1.4) |  |  |  |
| 54 | Electrical insulation of the tanker/terminal interface is effective (12.9.5, 17.4, 18.2.14) |  |  |  |
| 55 | Tank venting system and closed operation procedures are agreed (11.3.3.1, 21.4, 21.5, 23.3.3) |  |  |  |
| 56 | Vapor return line operational parameters are agreed (11.5, 18.3, 23.7.7) |  | N/A |  |
| 57 | Measures to avoid back-filling are agreed (12.1.13.7) |  |  |  |
| 58 | Status of unused cargo and bunker connections is satisfactory (23.7.1, 23.7.6) |  |  |  |
| 59 | Portable very high frequency and ultra high frequency radios are intrinsically safe (4.12.4, 21.1.1) |  |  |  |
| 60 | Procedures for receiving nitrogen from terminal to cargo tank are agreed (12.1.14.8) |  | N/A |  |

| ADDITIONAL FOR CHEMICAL TANKERS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Part 5B. Tanker and terminal: bulk liquid chemicals. Checks pre-transfer |  |  |  |  |
| Item | Check | Tanker status | Terminal status | Remarks |
| 61 | Inhibition certificate received (if required) from manufacturer |  | N/A |  |
| 62 | Appropriate personal protective equipment identified and available (4.8.1) |  | N/A |  |
| 63 | Countermeasures against personal contact with cargo are agreed (1.4) | A | N/A |  |
| 64 | Cargo handling rate and relationship with valve closure times and automatic shutdown systems is agreed (16.8, 21.4, 21.5, 21.6) | A | N/A |  |
| 65 | Cargo system gauge operation and alarm set points are confirmed (12.1.6.6.1) |  | N/A |  |

| Part 5B. Tanker and terminal: bulk liquid chemicals. Checks pre-transfer (cont.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Check | Tanker status | Terminal status | Remarks |
| 66 | Adequate portable vapor detection instruments are in use (2.4) |  | N/A |  |
| 67 | Information on firefighting media and procedures is exchanged (5, 19) |  | N/A |  |
| 68 | Transfer hoses confirmed suitable for the product being handled (18.2) |  | N/A |  |
| 69 | Confirm cargo handling is only by a permanent installed pipeline system |  | N/A |  |
| 70 | Procedures are in place to receive nitrogen from the terminal for inverting or purging (12.1.14.8) |  | N/A |  |

| ADDITIONAL FOR GAS TANKERS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Part 5C. Tanker and terminal: liquefied gas. Checks pre-transfer |  |  |  |  |
| Item | Check | Tanker status | Terminal status | Remarks |
| 71 | Inhibition certificate received (if required) from manufacturer |  | N/A |  |
| 72 | Water spray system is operational (5.3.1, 19.4.3) |  | N/A |  |
| 73 | Appropriate personal protective equipment is identified and available (4.8.1) |  | N/A |  |
| 74 | Remote control valves are operational |  | N/A |  |
| 75 | Cargo pumps and compressors are operational |  | N/A |  |
| 76 | Maximum working pressures are agreed between tanker and terminal (21.4, 21.5, 21.6) | A | N/A |  |
| 77 | Reliquefication or boil-off control equipment is operational |  | N/A |  |
| 78 | Gas detection equipment is appropriately set for the cargo (2.4) |  | N/A |  |
| 79 | Cargo system gauge operation and alarm set points are confirmed (12.1.6.6.1) |  | N/A |  |
| 80 | Emergency shutdown systems are tested and operational (18.5) |  | N/A |  |
| 81 | Cargo handling rate and relationship with valve closure times and automatic shutdown systems is agreed (16.8, 21.4, 21.5, 21.6) | A | N/A |  |
| 82 | Maximum/minimum temperatures/pressures of the cargo to be transferred are agreed (21.4, 21.5, 21.6) | A | N/A |  |
| 83 | Cargo tank relief valve settings are confirmed (12.11, 21.2, 21.4) |  | N/A |  |

| Part 6. Tanker and terminal: agreements pre-transfer |  |  |  |  |
| --- | --- | --- | --- | --- |
| Part 5 item | Agreement | Details | Tanker initials | Terminal initials |
| 32 | Tanker maneuvering readiness | Notice period (maximum) for full readiness to maneuver: Period of disablement (if permitted): |  | 60 Min |
| 33 | Security protocols | Security level: Local requirements: |  | 1 |
| 33 | Effective tanker/terminal communications | Primary system: Backup system: |  | Walkie Talky/ Phone and Cellphone Numbers /jetty man Pls see ship-shore agreement |
| 35 | Operational supervision and watchkeeping | Tanker: Terminal: |  | Jetty Man and Loading Master |
| 37 | Dedicated smoking areas and | Tanker: |  | N/A |
| 38 | naked lights restrictions | Terminal: |  |  |
| 45 | Maximum wind, current and sea/swell criteria or other environmental factors | Stop cargo transfer: Disconnect: Unberth: |  | 25 30 35 |
| 45 46 | Limits for cargo, bunkers and ballast handling | Maximum transfer rates: |  | 400 150 4 Pls See The Ship Shore Agreement bunkers operation not allowed |
|  |  | Topping-off rates: |  |  |
|  |  | Maximum manifold pressure: |  |  |
|  |  | Cargo temperature: |  |  |
|  |  | Other limitations: |  |  |
| 46 45 | Pressure surge control | Minimum number of cargo tanks open: |  | 2 150 N/A |
|  |  | Tank switching protocols: |  |  |
|  |  | Minimum number of cargo tanks open: |  |  |
|  |  | Tank switching protocols: |  |  |
|  |  | Full load rate: |  |  |
|  |  | Topping-off rate: |  |  |
|  |  | Closing time of automatic valves: |  |  |

| Part 6. Tanker and terminal: agreements pre-transfer (cont.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Part 5 item | Agreement | Details | Tanker initials | Terminal initials |
| 46 | Cargo transfer management procedures | Action notice periods: Transfer stop protocols: |  | 10min Stop Stop Stop |
| 50 | Routine for regular checks on cargo transferred are agreed | Routine transferred quantity checks: |  | Pls See the Ship Shore Agreement Every one hr |
| 51 | Emergency signals | Tanker: Terminal: STOP × 3 |  | STOP STOP STOP |
| 55 | Tank venting system | Procedure: |  | N/A |
| 55 | Closed operations | Requirements: |  |  |
| 56 | Vapour return line | Operational parameters: Maximum flow rate: |  | N/A |
| 60 | Nitrogen supply from terminal | Procedures to receive: Maximum pressure: Flow rate: |  | N/A |
| 83 | For gas tanker only: cargo tank relief valve settings | Tank 1: Tank 2: Tank 3: Tank 4: Tank 5: Tank 6: Tank 7: Tank 8: Tank 9: Tank 10: |  | N/A |
| XX | Exceptions and additions | Special issues that both parties should be aware of: |  |  |

| Part 7A. General tanker: checks pre-transfer |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 84 | Portable drip trays are correctly positioned and empty (23.7.5) |  |  |
| 85 | Individual cargo tank inert gas supply valves are secured for cargo plan (12.1.13.4) |  |  |
| 86 | Inert gas system delivering inert gas with oxygen content not more than 5% (11.1.3) |  |  |
| 87 | Cargo tank high level alarms are operational (12.1.6.6.1) |  |  |
| 88 | All cargo, ballast and bunker tanks openings are secured (23.3) |  |  |

| Part 7B. Tanker: checks pre-transfer if crude oil washing is planned |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 89 | The completed pre-arrival crude oil washing checklist, as contained in the approved crude oil washing manual, is copied to terminal (12.5.2, 21.2.3) |  | N/A |
| 90 | Crude oil washing checklists for use before, during and after crude oil washing are in place ready to complete, as contained in the approved crude oil washing manual (12.5.2, 21.6) |  | N/A |

# Checks after pre-transfer conference

## For tankers that will perform tank cleaning alongside and/or gas freeing alongside

| Part 7C. Tanker: checks prior to tank cleaning and/or gas freeing |  |  |  |
| --- | --- | --- | --- |
| Item | Check | Status | Remarks |
| 91 | Permission for tank cleaning operations is confirmed (21.2.3, 21.4, 25.4.3) |  |  |
| 92 | Permission for gas freeing operations is confirmed (12.4.3) |  |  |
| 93 | Tank cleaning procedures are agreed (12.3.2, 21.4, 21.6) |  |  |
| 94 | If cargo tank entry is required, procedures for entry have been agreed with the terminal (10.5) |  |  |
| 95 | Slop reception facilities and requirements are confirmed (12.1, 21.2, 21.4) |  |  |

Declaration

We the undersigned have checked the items in the applicable parts 1 to 7 as marked and signed below:

|  | Tanker | Terminal |
| --- | --- | --- |
| Part 1A. Tanker: checks pre-arrival |  |  |
| Part 1B. Tanker: checks pre-arrival if using an inert gas system |  |  |
| Part 2. Terminal: checks pre-arrival |  |  |
| Part 3. Tanker: checks after mooring |  |  |
| Part 4. Terminal: checks after mooring |  |  |
| Part 5A. Tanker and terminal: pre-transfer conference |  |  |
| Part 5B. Tanker and terminal: bulk liquid chemicals. Checks pre-transfer |  | N/A |
| Part 5C. Tanker and terminal: liquefied gas. Checks pre-transfer |  | N/A |
| Part 6. Tanker and terminal: agreements pre-transfer |  |  |
| Part 7A. General tanker: checks pre-transfer |  |  |
| Part 7B. Tanker: checks pre-transfer if crude oil washing is planned |  | N/A |
| Part 7C. Tanker: checks prior to tank cleaning and/or gas freeing |  | N/A |

In accordance with the guidance in chapter 25 of ISGOTT, we have satisfied ourselves that the entries we have made are correct to the best of our knowledge and that the tanker and terminal are in agreement to undertake the transfer operation.

We have also agreed to carry out the repetitive checks noted in parts 9 and 10 of the ISGOTT SSSCL, which should occur at intervals of not more than......4.......hours for the tanker and not more than.....4......hours for the terminal.

If, to our knowledge, the status of any item changes, we will immediately inform the other party.

| Tanker | Terminal |
| --- | --- |
| Name : | Name : |
| Rank : | Position : LOADING MASTER |
| Signature: | Signature : |
| Date : | Date : |
| Time : | Time : |

# Checks during transfer

## Repetitive checks

| Part 8. Tanker: repetitive checks during and after transfer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item ref | Check | DATE | DATE | DATE | DATE | DATE | DATE | DATE | DATE | DATE | DATE | DATE | DATE | Remarks |
|  |  | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time | Time |  |
| Interval time: 4 hrs. |  | 08.03 | 08.03 | 08.03 | 08.04 | 08.04 | 08.04 | 08.05 | 08.05 | 08.05 | 08.06 | 08.06 | 08.06 |  |
|  |  | 03:00 | 11:00 | 19:00 | 03:00 | 11:00 | 19:00 | 03:00 | 11:00 | 19:00 | 03:00 | 11:00 | 19:00 |  |
| Interval time: 4 hrs. |  | 08.03 | 08.03 | 08.03 | 08.04 | 08.04 | 08.04 | 08.05 | 08.05 | 08.05 | 08.06 | 08.06 | 08.06 |  |
|  |  | 07:00 | 15:00 | 23:00 | 07:00 | 15:00 | 23:00 | 07:00 | 15:00 | 23:00 | 07:00 | 15:00 | 23:00 |  |
| 8 | Inert gas system pressure and oxygen recording operational | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A |  |
| 9 | Inert gas system and all associated equipment are operational | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A | × N/A |  |
| 11 | Cargo tank atmospheres are at positive pressure | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 18 | Mooring arrangement is effective | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 19 | Access to and from the tanker is safe | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 20 | Scuppers and save-alls are plugged | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 23 | External openings in superstructures are controlled | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 24 | Pumproom ventilation is effective | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 28 | Tanker is ready to move at agreed notice period | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 29 | Fendering is effective | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 33 | Communications are effective | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 35 | Supervision and watchkeeping is adequate | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 36 | Sufficient personnel are available to deal with an emergency | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 37 | Smoking restrictions and designated smoking areas are complied with | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 38 | Naked light restrictions are complied with | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |

| Part 8. Tanker: repetitive checks during and after transfer (cont.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | Control of electrical devices and equipment in hazardous zones is complied with | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 40 41 42 51 | Emergency response preparedness is satisfactory | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 54 | Electrical insulation of the tanker/terminal interface is effective | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 55 | Tank venting system and closed operation procedures are as agreed | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 85 | Individual cargo tank inert gas valves settings are as agreed | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 86 | Inert gas delivery maintained at not more than 5% oxygen | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |
| 87 | Cargo tank high level alarms are operational | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES | YES |  |

| Initials |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initials |  |  |  |  |  |  |  |  |  |  |  |  |

| DISTRIBUTION: Original: TERMINAL Copy: Ship’s file | UNIFIED FILING SYSTEM: File No: 8 |
| --- | --- |